### I2C Touch Sensor Module Connection

Unlike simple analog touch sensors, I2C touch modules (like the MPR121) communicate digitally, allowing you to manage multiple touch pads (up to 12+) using only two signal wires.

### Connection Details

- VCC: Connect to the microcontroller's power output (usually 3.3V or 5V, depending on the module's datasheet).

- GND: Connect to GND.

- SDA (Serial Data): Connect to the SDA pin on your microcontroller.

- SCL (Serial Clock): Connect to the SCL pin on your microcontroller.

### Important Considerations

- Addressing: Each I2C device has a unique address. If you connect multiple I2C sensors to the same bus, ensure they have different addresses or use an I2C multiplexer.

- Pull-up Resistors: I2C lines require pull-up resistors (typically 2.2kΩ to 10kΩ). Most breakout boards have these built-in; if yours does not, you must add them between SDA/SCL and VCC.

- Libraries: Since I2C communication is more complex than analog reading, you will need a sensor-specific library (e.g., Adafruit_MPR121 for the MPR121 module). Search for the specific sensor model in the Arduino Library Manager to install the relevant driver.

### Pro-Tip: The "I2C Scanner"

If you are unsure if your sensor is connected properly, upload an "I2C Scanner" sketch to your Arduino. This code will scan the I2C bus and print the hexadecimal address of any detected device to your Serial Monitor, confirming your wiring is correct.