Important

Before uploading the code:

- Open Arduino IDE

- Go to:

Sketch → Include Library → Manage Libraries

- Search:

DHT sensor library

- Install:

  - DHT sensor library by Adafruit

  - Adafruit Unified Sensor

- Connections (DHT11 → Arduino UNO)

| DHT11 | Arduino |
| --- | --- |
| VCC | 5V |
| DATA | D2 |
| NC | Leave empty |
| GND | GND |
| Add Pull-up Resistor (Important) If you are using the bare DHT11 sensor (not module), connect: 10kΩ resistor between VCC and DATA Without this resistor, the sensor often fails. |  |