### Force Sensor (FSR) Connection

Force Sensitive Resistors (FSRs) operate as a variable resistor. Unlike the MQ series, they do not require an external power source for the sensor itself; instead, they require a Voltage Divider circuit to convert resistance changes into voltage changes for your microcontroller.

- Pin 1 (Sensor): Connect to 5V.

- Pin 2 (Sensor): Connect to an Analog Input Pin (e.g., A0) AND to a 10kΩ resistor (pull-down resistor).

- 10kΩ Resistor: Connect the other end of this resistor to GND.

### Required Components & Libraries

- Resistor: You must use a 10kΩ resistor to create the voltage divider, otherwise, the analog pin will float and provide unstable data.

- Libraries: No external libraries are needed. You simply use the standard analogRead() function in your code to get values ranging from 0–1023.

### How it works

- When no pressure is applied, the FSR has very high resistance, and the Analog pin reads 0.

- As pressure is applied, the resistance drops, allowing more voltage to pass to the Analog pin, resulting in a higher value (up to 1023).