### MQ-135 Pinout Connection (Air Quality/Hazardous Gas)

The MQ-135 sensor uses the same standard 4-pin interface as the MQ-7 and MQ-5. Connect it to your microcontroller (e.g., Arduino) as follows:

- VCC: Connect to 5V.

- GND: Connect to GND.

- AOUT: Connect to an Analog Input Pin (e.g., A0) for real-time air quality/gas concentration data.

- DOUT: Connect to a Digital Input Pin (e.g., D2) for simple high/low alerts, adjusted via the onboard potentiometer.

### Required Libraries

- MQUnifiedsensor: Highly recommended for this specific sensor to perform the necessary pre-heating and to calibrate the sensor according to its specific sensitivity curves for gases like Ammonia, $CO_2$, and Alcohol.

- Note: As with the others, no library is strictly necessary if you are only reading raw analog values (0–1023) for threshold triggers.

### Important Note

The MQ-135 requires a "burn-in" period of at least 24 hours of continuous power to achieve reliable readings, as the heating element must stabilize to sense ambient air quality accurately.