### TTP233 Touch Sensor Module Connection

The TTP233 is a capacitive touch sensor. It functions like a button—when you touch the pad, the output pin changes state.

### Pinout Connection

- VCC: Connect to 3.3V or 5V (The module supports a wide voltage range).

- GND: Connect to GND.

- SIG / OUT: Connect to a Digital Input Pin (e.g., D2) on your microcontroller.

### Required Libraries

- None: This sensor acts as a simple digital switch. You do not need any special libraries. Simply use digitalRead(pin) in your code to detect the touch (it will return HIGH or LOW depending on the state).

### Key Technical Notes

- Output Logic: By default, the output is usually LOW and goes HIGH when touched.

- Soldering Jumpers: Many TTP233 modules have small A and B solder pads on the back. These allow you to change the output mode (e.g., toggle mode vs. momentary mode). If you need the sensor to act like a light switch (staying ON until touched again), you may need to bridge these pads.