### IR Tracking Sensor (Line Follower/Proximity) Connection

Tracking sensors (commonly IR-based modules like the TCRT5000 or KY-033) are typically used for line-following robots or object detection.

- VCC: Connect to 5V.

- GND: Connect to GND.

- OUT (or DO): Connect to any Digital Input Pin (e.g., D2) on your microcontroller.

- AO (Optional): Connect to an Analog Input Pin (e.g., A0) if you need precise reflection intensity data rather than just a simple high/low detection.

### Required Libraries

- None: These sensors output a simple TTL signal (Digital) or voltage level (Analog) that can be read directly using standard commands like digitalRead() or analogRead(). No external libraries are necessary for basic operation.

### Key Tips

- Sensitivity: Most modules have a small onboard potentiometer (trimmer). Use a small screwdriver to turn it to adjust the sensitivity threshold, which determines the distance at which the sensor detects a surface or object.

- Mounting: For line tracking, the sensor should be mounted roughly 5mm to 10mm above the surface for the best performance.

- Interference: These sensors are sensitive to ambient light. If you are getting inconsistent readings, ensure the sensor is shielded from direct sunlight or bright overhead lamps.